| KABO TOOL COMPANY TW1 - Calibration SOP |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| maintain |  | Calibration SOP |  |  | Date： |  | version no： |
|  |  |  |  |  | 2015/01/30 |  | A1 |
|  |  |  |  |  | Establish date : |  | PAGE/OF： |
|  |  |  |  |  | 2015/01/30 |  | 1/2 |
| Process |  |  | Photo |  |  | Operation/Testing points |  |
| ` Preparation |  |  |  |  |  | - Torque Tester: - Torque tester- KABO_KTT1500 - Torque loader- Norbar_I.S.O.2000 - Transducer (500 NM)- Norbar torque - Product - TW1 Torque Wrench - Tools - 6-Point 22mm Socket - 3/8”Dr. Ratchet handle - 3/4" 四角條 - Level |  |
| 20% of the max torque Calibration |  |  | 3-2 2-2 |  |  | To confirm the initial scale- - Use the torque equipment to check the accuracy, 150 Ft-lb. - Place the torque wrench on the torque loader and use “level” to check the wrench in horizontal line. - Adjust the forcing point in place. - Make sure the value 150 Ft-lb within +/-4% tolerance. 3-2 The scale must be “face-up”. |  |
| Authorize | Plant Manager: Edward Chang |  |  | QC Manager：Jack Tsai |  | Issue by Ben Chang |  |
| KABO TOOL COMPANY TW1 – Calibration SOP |  |  |  |  |  |  |  |
| maintain |  | Calibration SOP |  |  | Date： |  | Version no: |
|  |  |  |  |  | 2015/01/30 |  | A1 |
|  |  |  |  |  | Establish date : |  | PAGE/OF： |
|  |  |  |  |  | 2015/01/30 |  | 2/2 |
| Process |  |  | Photo |  |  | Operation/Testing points |  |
| 60% of the max torque Calibration |  |  |  |  |  | Check the torque value of 350 Ft-lb- - When 150 Ft-lb values confirmed, then turn 18.5 laps clock-wisely to 350 Ft-lb. - Then, check the value 350 Ft-lb is within +/-4% tolerance. |  |
| 100% of the max torque Calibration |  |  |  |  |  | Check the torque value of 600 Ft-lb- - To turn the handle 22.5 laps clock-wisely to 600 Ft-lb. - Then, check the value 600 Ft-lb is within +/-4% tolerance. - Once the toque value is within +/-4% tolerance, then turn 41 laps counter-clockwisely to 150 Ft-lb. |  |
| Authorize | Plant Manager: Edward Chang |  |  | QC Manager：Jack Tsai |  | Issue by Ben Chang |  |